Program GEODESIC for setting Waypoints in ATP

Version 1.0

(C) G. Berendt, 09/1992

Given the coordinates of two points in ATP, this program calculates the geo-

desic path (path of shortest length) between the points and displays the co-

ordinates of waypoints, which lie on the geodesic in equal distances. In ad-

dition to this, the program calculates the distances between the waypoints

along lines, which cut the meridians under constant angles (Loxodromes) as

well as these angles.

The program can be used to set an arbitrary number of such waypoints for in-

stance along a route from the U.S.A. to Great Britain. As the magnetic devi-

ation within the ATP default scenery (or the East-scenery in IPS) a priori

is not known and moreover does not smoothly change to the corresponding

deviation in the Great Britain Scenery, the angles displayed are only a very

rough approximation to the measured ones and there will be a jump in the

course at a longitude of about 45� W.

Hence, you should only use the calculated angles as a guideline between those

two waypoints which lie on either side of the jump in the sceneries, because

it is only possible to set waypoints by menu F7, Option 5, within the same

scenery. For this reason it is also recommended to choose two waypoints at

least at approximately 46� W and 43� W longitude along the course.

With the precautions mentioned, it is possible to cross the Atlantic in ATP

with a B 747 or a B 767 without further difficulties.

Here follows as an example the outprint of the route file between the airports

of Boston, MA and Shannon, IR:

Waypoints for the Route BOS - INN

Latitude 1:   42� 21' 0"

Longitude 1:  71� 0' 0"

Latitude 2:   52� 42' 0"

Longitude 2:  8� 56' 0"

Geodesic Distance: 2508.29 NM

-----------------------

Number of Sections: 8

Waypoint 1:

Lat[1]:                           45� 18' 32"

Lon[1]:                           65�  1' 49"

Geodesic Distance[1]:               313.54 NM

Constant Course Distance[1]:        313.60 NM

True Geographic Course Angle[1]:     55.50�

Total Constant Course Distance:     313.60 NM

Waypoint 2:

Lat[2]:                           47� 55' 27"

Lon[2]:                           58� 26' 38"

Geodesic Distance[2]:               313.54 NM

Constant Course Distance[2]:        313.63 NM

True Geographic Course Angle[2]:     59.96�

Total Constant Course Distance:     627.23 NM

Waypoint 3:

Lat[3]:                           50�  7' 30"

Lon[3]:                           51� 12' 56"

Geodesic Distance[3]:               313.54 NM

Constant Course Distance[3]:        313.66 NM

True Geographic Course Angle[3]:     65.09�

Total Constant Course Distance:     940.89 NM

Waypoint 4:

Lat[4]:                           51� 50' 13"

Lon[4]:                           43� 22' 22"

Geodesic Distance[4]:               313.54 NM

Constant Course Distance[4]:        313.68 NM

True Geographic Course Angle[4]:     70.87�

Total Constant Course Distance:    1254.57 NM

Waypoint 5:

Lat[5]:                           52� 59' 27"

Lon[5]:                           35�  0' 58"

Geodesic Distance[5]:               313.54 NM

Constant Course Distance[5]:        313.71 NM

True Geographic Course Angle[5]:     77.24�

Total Constant Course Distance:    1568.28 NM

Waypoint 6:

Lat[6]:                           53� 31' 57"

Lon[6]:                           26� 19' 33"

Geodesic Distance[6]:               313.54 NM

Constant Course Distance[6]:        313.73 NM

True Geographic Course Angle[6]:     84.05�

Total Constant Course Distance:    1882.01 NM

Waypoint 7:

Lat[7]:                           53� 26'  1"

Lon[7]:                           17� 32' 41"

Geodesic Distance[7]:               313.54 NM

Constant Course Distance[7]:        313.75 NM

True Geographic Course Angle[7]:     91.08�

Total Constant Course Distance:    2195.76 NM

Waypoint 8:

Lat[8]:                           52� 42'  0"

Lon[8]:                            8� 55' 60"

Geodesic Distance[8]:               313.54 NM

Constant Course Distance[8]:        313.73 NM

True Geographic Course Angle[8]:     98.07�

Total Constant Course Distance:    2509.49 NM

Actually, the jump between the US Scenery and the GB Scenery happens somewhere

between Waypoint 3 and Waypoint 4. You should either divide that section once

more or use the given course until the jump has occured and then adjust it

accordingly.

The author declines any responsibility for damages or inconveniencies the pro-

gram might cause; but you may use the program freely.

G. Berendt

[CIS 100112,2506], [Internet: berendt@math.fu-berlin.de]